Whole Structure Isometric (Square Section)

Job Information

|  | Engineer | Checked | Approved |
| --- | --- | --- | --- |
| Name: |  |  |  |
| Date: |  |  |  |

| Structure Type | SPACE FRAME |
| --- | --- |

| Number of Nodes | 4 | Highest Node | 4 |
| --- | --- | --- | --- |
| Number of Elements | 3 | Highest Beam | 3 |

| Number of Basic Load Cases | 1 |
| --- | --- |
| Number of Combination Load Cases | 0 |

Included in this printout are data for:

| All | The Whole Structure |
| --- | --- |

Included in this printout are results for load cases:

| Type | L/C | Name |
| --- | --- | --- |
| Primary | 1 | LOAD CASE 1 |

Nodes

| Node | X (in) | Y (in) | Z (in) |
| --- | --- | --- | --- |
| 1 | 0.000 | 0.000 | 0.000 |
| 2 | 0.000 | 100.000 | 0.000 |
| 3 | 100.000 | 100.000 | 0.000 |
| 4 | 100.000 | 0.000 | 0.000 |

Beams

| Beam | Node A | Node B | Length (in) | Property |  (degrees) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 100.000 | 2 | 0 |
| 2 | 2 | 3 | 100.000 | 2 | 0 |
| 3 | 3 | 4 | 100.000 | 2 | 0 |

Section Properties

| Prop | Section | Area (in2) | Iyy (in4) | Izz (in4) | J (in4) | Material |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IPE100 | 1.596 | 0.384 | 4.108 | 0.022 | STEEL |
| 2 | Rect 5.00x5.00 | 25.000 | 52.083 | 52.083 | 87.891 | STEEL |

Materials

| Mat | Name | E (kip/in2) |  | Density (kip/in3) |  (1/°F) |
| --- | --- | --- | --- | --- | --- |
| 3 | STEEL | 29E 3 | 0.300 | 0.000 | 6.5E -6 |
| 4 | STAINLESSSTEEL | 28E 3 | 0.300 | 0.000 | 9.9E -6 |
| 5 | ALUMINUM | 10E 3 | 0.330 | 0.000 | 12.8E -6 |
| 6 | CONCRETE | 3.15E 3 | 0.170 | 0.000 | 5.5E -6 |

Supports

| Node | X (kip/in) | Y (kip/in) | Z (kip/in) | rX (kip-ft/deg) | rY (kip-ft/deg) | rZ (kip-ft/deg) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Fixed | Fixed | Fixed | Fixed | Fixed | Fixed |
| 4 | Fixed | Fixed | Fixed | Fixed | Fixed | Fixed |

Releases

There is no data of this type.

Basic Load Cases

| Number | Name |
| --- | --- |
| 1 | LOAD CASE 1 |

Combination Load Cases

There is no data of this type.

Load Generators

There is no data of this type.

Node Loads : 1 LOAD CASE 1

| Node | FX (lb) | FY (lb) | FZ (lb) | MX (lb-in) | MY (lb-in) | MZ (lb-in) |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 500.000 | - | - | - | - | - |

Node Displacements

| Node | L/C | X (in) | Y (in) | Z (in) | Resultant (in) | rX (rad) | rY (rad) | rZ (rad) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| 2 | 1:LOAD CASE 1 | 0.020 | 0.000 | 0.000 | 0.020 | 0.000 | 0.000 | -0.000 |
| 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 | 0.000 | 0.000 | -0.000 |
| 4 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

Node Displacement Summary

|  | Node | L/C | X (in) | Y (in) | Z (in) | Resultant (in) | rX (rad) | rY (rad) | rZ (rad) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max X | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 | 0.000 | 0.000 | -0.000 |
| Min X | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Max Y | 2 | 1:LOAD CASE 1 | 0.020 | 0.000 | 0.000 | 0.020 | 0.000 | 0.000 | -0.000 |
| Min Y | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 | 0.000 | 0.000 | -0.000 |
| Max Z | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Min Z | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Max rX | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Min rX | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Max rY | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Min rY | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Max rZ | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Min rZ | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 | 0.000 | 0.000 | -0.000 |
| Max Rst | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 | 0.000 | 0.000 | -0.000 |

Beam Displacement Detail

Displacements shown in italic indicate the presence of an offset

| Beam | L/C | d (in) | X (in) | Y (in) | Z (in) | Resultant (in) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  | 10.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  | 20.000 | 0.002 | 0.000 | 0.000 | 0.002 |
|  |  | 30.000 | 0.004 | 0.000 | 0.000 | 0.004 |
|  |  | 40.000 | 0.006 | 0.000 | 0.000 | 0.006 |
|  |  | 50.000 | 0.008 | 0.000 | 0.000 | 0.008 |
|  |  | 60.000 | 0.011 | 0.000 | 0.000 | 0.011 |
|  |  | 70.000 | 0.014 | 0.000 | 0.000 | 0.014 |
|  |  | 80.000 | 0.016 | 0.000 | 0.000 | 0.016 |
|  |  | 90.000 | 0.018 | 0.000 | 0.000 | 0.018 |
|  |  | 100.000 | 0.020 | 0.000 | 0.000 | 0.020 |
| 2 | 1:LOAD CASE 1 | 0.000 | 0.020 | 0.000 | 0.000 | 0.020 |
|  |  | 10.000 | 0.020 | -0.001 | 0.000 | 0.020 |
|  |  | 20.000 | 0.020 | -0.001 | 0.000 | 0.020 |
|  |  | 30.000 | 0.020 | -0.001 | 0.000 | 0.020 |
|  |  | 40.000 | 0.020 | -0.001 | 0.000 | 0.020 |
|  |  | 50.000 | 0.020 | 0.000 | 0.000 | 0.020 |
|  |  | 60.000 | 0.020 | 0.001 | 0.000 | 0.020 |
|  |  | 70.000 | 0.020 | 0.001 | 0.000 | 0.020 |
|  |  | 80.000 | 0.020 | 0.001 | 0.000 | 0.020 |
|  |  | 90.000 | 0.020 | 0.001 | 0.000 | 0.020 |
|  |  | 100.000 | 0.020 | -0.000 | 0.000 | 0.020 |
| 3 | 1:LOAD CASE 1 | 0.000 | 0.020 | -0.000 | 0.000 | 0.020 |
|  |  | 10.000 | 0.018 | -0.000 | 0.000 | 0.018 |
|  |  | 20.000 | 0.016 | -0.000 | 0.000 | 0.016 |
|  |  | 30.000 | 0.014 | -0.000 | 0.000 | 0.014 |
|  |  | 40.000 | 0.011 | -0.000 | 0.000 | 0.011 |
|  |  | 50.000 | 0.008 | -0.000 | 0.000 | 0.008 |
|  |  | 60.000 | 0.006 | -0.000 | 0.000 | 0.006 |
|  |  | 70.000 | 0.004 | -0.000 | 0.000 | 0.004 |
|  |  | 80.000 | 0.002 | -0.000 | 0.000 | 0.002 |
|  |  | 90.000 | 0.000 | -0.000 | 0.000 | 0.000 |
|  |  | 100.000 | 0.000 | 0.000 | 0.000 | 0.000 |

Beam Displacement Detail Summary

Displacements shown in italic indicate the presence of an offset

|  | Beam | L/C | d (in) | X (in) | Y (in) | Z (in) | Resultant (in) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Max X | 2 | 1:LOAD CASE 1 | 100.000 | 0.020 | -0.000 | 0.000 | 0.020 |
| Min X | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Max Y | 2 | 1:LOAD CASE 1 | 80.000 | 0.020 | 0.001 | 0.000 | 0.020 |
| Min Y | 2 | 1:LOAD CASE 1 | 20.000 | 0.020 | -0.001 | 0.000 | 0.020 |
| Max Z | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Min Z | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Max Rst | 2 | 1:LOAD CASE 1 | 80.000 | 0.020 | 0.001 | 0.000 | 0.020 |

Beam Relative Displacement Detail

| Beam | L/C | d (in) | Disp. x x (in) | Disp. y y (in) | Disp. z z (in) | Resultant (in) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  | 10.000 | -0.001 | -0.000 | 0.000 | 0.001 |
|  |  | 20.000 | -0.002 | -0.000 | 0.000 | 0.002 |
|  |  | 30.000 | -0.002 | -0.000 | 0.000 | 0.002 |
|  |  | 40.000 | -0.002 | -0.000 | 0.000 | 0.002 |
|  |  | 50.000 | -0.001 | -0.000 | 0.000 | 0.001 |
|  |  | 60.000 | -0.001 | -0.000 | 0.000 | 0.001 |
|  |  | 70.000 | -0.000 | -0.000 | 0.000 | 0.000 |
|  |  | 80.000 | 0.000 | -0.000 | 0.000 | 0.000 |
|  |  | 90.000 | 0.000 | -0.000 | 0.000 | 0.000 |
|  |  | 100.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| 2 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  | 10.000 | 0.000 | -0.001 | 0.000 | 0.001 |
|  |  | 20.000 | 0.000 | -0.001 | 0.000 | 0.001 |
|  |  | 30.000 | 0.000 | -0.001 | 0.000 | 0.001 |
|  |  | 40.000 | 0.000 | -0.001 | 0.000 | 0.001 |
|  |  | 50.000 | -0.000 | 0.000 | 0.000 | 0.000 |
|  |  | 60.000 | -0.000 | 0.001 | 0.000 | 0.001 |
|  |  | 70.000 | -0.000 | 0.001 | 0.000 | 0.001 |
|  |  | 80.000 | -0.000 | 0.001 | 0.000 | 0.001 |
|  |  | 90.000 | -0.000 | 0.001 | 0.000 | 0.001 |
|  |  | 100.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| 3 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  | 10.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  | 20.000 | 0.000 | -0.000 | 0.000 | 0.000 |
|  |  | 30.000 | -0.000 | -0.000 | 0.000 | 0.000 |
|  |  | 40.000 | -0.001 | -0.000 | 0.000 | 0.001 |
|  |  | 50.000 | -0.001 | -0.000 | 0.000 | 0.001 |
|  |  | 60.000 | -0.002 | 0.000 | 0.000 | 0.002 |
|  |  | 70.000 | -0.002 | -0.000 | 0.000 | 0.002 |
|  |  | 80.000 | -0.002 | -0.000 | 0.000 | 0.002 |
|  |  | 90.000 | -0.001 | -0.000 | 0.000 | 0.001 |
|  |  | 100.000 | 0.000 | 0.000 | 0.000 | 0.000 |

Beam Maximum Relative Displacements

Distances to maxima are given from beam end A.

| Beam | Node A | Length (in) | L/C | y (in) | d (in) | z (in) | d (in) | Resultant (in) | d (in) | Span Max z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 100.000 | 1:LOAD CASE 1 | -0.000 | 91.667 | 0.000 | 0.000 | 0.002 | 25.000 | > 10000 |
| 2 | 2 | 100.000 | 1:LOAD CASE 1 | 0.001 | 75.000 | 0.000 | 0.000 | 0.001 | 75.000 | > 10000 |
| 3 | 3 | 100.000 | 1:LOAD CASE 1 | -0.000 | 33.333 | 0.000 | 0.000 | 0.002 | 75.000 | > 10000 |

Beam End Displacements

Displacements shown in italic indicate the presence of an offset

| Beam | Node | L/C | X (in) | Y (in) | Z (in) | Resultant (in) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 |
|  | 2 | 1:LOAD CASE 1 | 0.020 | 0.000 | 0.000 | 0.020 |
| 2 | 2 | 1:LOAD CASE 1 | 0.020 | 0.000 | 0.000 | 0.020 |
|  | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 |
| 3 | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 |
|  | 4 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 |

Beam End Displacement Summary

Displacements shown in italic indicate the presence of an offset

|  | Beam | Node | L/C | X (in) | Y (in) | Z (in) | Resultant (in) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Max X | 2 | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 |
| Min X | 1 | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 |
| Max Y | 1 | 2 | 1:LOAD CASE 1 | 0.020 | 0.000 | 0.000 | 0.020 |
| Min Y | 2 | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 |
| Max Z | 1 | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 |
| Min Z | 1 | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 | 0.000 |
| Max Rst | 2 | 3 | 1:LOAD CASE 1 | 0.020 | -0.000 | 0.000 | 0.020 |

Beam End Forces

Sign convention is as the action of the joint on the beam.

|  |  |  | Axial | Shear |  | Torsion | Bending |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beam | Node | L/C | Fx (lb) | Fy (lb) | Fz (lb) | Mx (lb-in) | My (lb-in) | Mz (lb-in) |
| 1 | 1 | 1:LOAD CASE 1 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
|  | 2 | 1:LOAD CASE 1 | 213.899 | -249.844 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
| 2 | 2 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
|  | 3 | 1:LOAD CASE 1 | 249.844 | 213.899 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
| 3 | 3 | 1:LOAD CASE 1 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
|  | 4 | 1:LOAD CASE 1 | -213.899 | -250.156 | 0.000 | 0.000 | 0.000 | 14.3E 3 |

Beam End Force Summary

The signs of the forces at end B of each beam have been reversed. For  example: this means that the Min Fx entry gives the largest tension value for an beam.

|  |  |  |  | Axial | Shear |  | Torsion | Bending |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Beam | Node | L/C | Fx (lb) | Fy (lb) | Fz (lb) | Mx (lb-in) | My (lb-in) | Mz (lb-in) |
| Max Fx | 3 | 3 | 1:LOAD CASE 1 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
| Min Fx | 2 | 2 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
| Max Fy | 3 | 3 | 1:LOAD CASE 1 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
| Min Fy | 2 | 2 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
| Max Fz | 1 | 1 | 1:LOAD CASE 1 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min Fz | 1 | 1 | 1:LOAD CASE 1 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max Mx | 1 | 1 | 1:LOAD CASE 1 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min Mx | 1 | 1 | 1:LOAD CASE 1 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max My | 1 | 1 | 1:LOAD CASE 1 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min My | 1 | 1 | 1:LOAD CASE 1 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max Mz | 1 | 1 | 1:LOAD CASE 1 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min Mz | 3 | 4 | 1:LOAD CASE 1 | 213.899 | 250.156 | -0.000 | -0.000 | -0.000 | -14.3E 3 |

Beam End Forces Envelope

Sign convention is as the action of the joint on the beam.

| Beam | Node | Envelope | Fx (lb) | Fy (lb) | Fz (lb) | Mx (lb-in) | My (lb-in) | Mz (lb-in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | +ve | 0.000 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
|  |  |  | - | 1:LOAD CASE 1 | - | - | - | 1:LOAD CASE 1 |
|  |  |  | -213.899 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  |  | 1:LOAD CASE 1 | - | - | - | - | - |
|  | 2 | -ve | 0.000 | 249.844 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  |  | - | 1:LOAD CASE 1 | - | - | - | - |
|  |  |  | -213.899 | 0.000 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
|  |  |  | 1:LOAD CASE 1 | - | - | - | - | 1:LOAD CASE 1 |
| 2 | 2 | +ve | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  |  | - | - | - | - | - | - |
|  |  |  | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
|  |  |  | 1:LOAD CASE 1 | 1:LOAD CASE 1 | - | - | - | 1:LOAD CASE 1 |
|  | 3 | -ve | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
|  |  |  | - | - | - | - | - | 1:LOAD CASE 1 |
|  |  |  | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  |  | 1:LOAD CASE 1 | 1:LOAD CASE 1 | - | - | - | - |
| 3 | 3 | +ve | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
|  |  |  | 1:LOAD CASE 1 | 1:LOAD CASE 1 | - | - | - | 1:LOAD CASE 1 |
|  |  |  | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  |  | - | - | - | - | - | - |
|  | 4 | -ve | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 0.000 |
|  |  |  | 1:LOAD CASE 1 | 1:LOAD CASE 1 | - | - | - | - |
|  |  |  | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | -14.3E 3 |
|  |  |  | - | - | - | - | - | 1:LOAD CASE 1 |

Beam Force Detail

Sign convention as diagrams:- positive above line, negative below line except Fx where positive is compression. Distance d is given from beam end A.

|  |  |  | Axial | Shear |  | Torsion | Bending |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beam | L/C | d (in) | Fx (lb) | Fy (lb) | Fz (lb) | Mx (lb-in) | My (lb-in) | Mz (lb-in) |
| 1 | 1:LOAD CASE 1 | 0.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
|  |  | 10.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 11.8E 3 |
|  |  | 20.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 9.3E 3 |
|  |  | 30.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 6.8E 3 |
|  |  | 40.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 4.3E 3 |
|  |  | 50.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 1.8E 3 |
|  |  | 60.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | -696.001 |
|  |  | 70.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | -3.19E 3 |
|  |  | 80.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | -5.69E 3 |
|  |  | 90.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | -8.19E 3 |
|  |  | 100.000 | -213.899 | 249.844 | -0.000 | -0.000 | -0.000 | -10.7E 3 |
| 2 | 1:LOAD CASE 1 | 0.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
|  |  | 10.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -8.55E 3 |
|  |  | 20.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -6.41E 3 |
|  |  | 30.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -4.27E 3 |
|  |  | 40.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -2.13E 3 |
|  |  | 50.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 5.185 |
|  |  | 60.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 2.14E 3 |
|  |  | 70.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 4.28E 3 |
|  |  | 80.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 6.42E 3 |
|  |  | 90.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 8.56E 3 |
|  |  | 100.000 | -249.844 | -213.899 | -0.000 | -0.000 | -0.000 | 10.7E 3 |
| 3 | 1:LOAD CASE 1 | 0.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
|  |  | 10.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 8.2E 3 |
|  |  | 20.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 5.7E 3 |
|  |  | 30.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 3.2E 3 |
|  |  | 40.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 693.926 |
|  |  | 50.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | -1.81E 3 |
|  |  | 60.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | -4.31E 3 |
|  |  | 70.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | -6.81E 3 |
|  |  | 80.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | -9.31E 3 |
|  |  | 90.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | -11.8E 3 |
|  |  | 100.000 | 213.899 | 250.156 | -0.000 | -0.000 | -0.000 | -14.3E 3 |

Beam Force Detail Summary

Sign convention as diagrams:- positive above line, negative below line except Fx where positive is compression. Distance d is given from beam end A.

|  |  |  |  | Axial | Shear |  | Torsion | Bending |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Beam | L/C | d (in) | Fx (lb) | Fy (lb) | Fz (lb) | Mx (lb-in) | My (lb-in) | Mz (lb-in) |
| Max Fx | 3 | 1:LOAD CASE 1 | 0.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
| Min Fx | 2 | 1:LOAD CASE 1 | 0.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
| Max Fy | 3 | 1:LOAD CASE 1 | 0.000 | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 10.7E 3 |
| Min Fy | 2 | 1:LOAD CASE 1 | 0.000 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -10.7E 3 |
| Max Fz | 1 | 1:LOAD CASE 1 | 0.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min Fz | 1 | 1:LOAD CASE 1 | 0.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max Mx | 1 | 1:LOAD CASE 1 | 0.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min Mx | 1 | 1:LOAD CASE 1 | 0.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max My | 1 | 1:LOAD CASE 1 | 0.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min My | 1 | 1:LOAD CASE 1 | 0.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max Mz | 1 | 1:LOAD CASE 1 | 0.000 | -213.899 | 249.844 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min Mz | 3 | 1:LOAD CASE 1 | 100.000 | 213.899 | 250.156 | -0.000 | -0.000 | -0.000 | -14.3E 3 |

Beam Maximum Moments

Distances to maxima are given from beam end A.

| Beam | Node A | Length (in) | L/C |  | d (in) | Max My (lb-in) | d (in) | Max Mz (lb-in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 100.000 | 1:LOAD CASE 1 | Max -ve | 0.000 | 0.000 | 0.000 | 14.3E 3 |
|  |  |  |  | Max +ve | 0.000 | 0.000 | 100.000 | -10.7E 3 |
| 2 | 2 | 100.000 | 1:LOAD CASE 1 | Max -ve | 0.000 | 0.000 | 100.000 | 10.7E 3 |
|  |  |  |  | Max +ve | 0.000 | 0.000 | 0.000 | -10.7E 3 |
| 3 | 3 | 100.000 | 1:LOAD CASE 1 | Max -ve | 0.000 | 0.000 | 0.000 | 10.7E 3 |
|  |  |  |  | Max +ve | 0.000 | 0.000 | 100.000 | -14.3E 3 |

Beam Maximum Shear Forces

Distances to maxima are given from beam end A.

| Beam | Node A | Length (in) | L/C |  | d (in) | Max Fz (lb) | d (in) | Max Fy (lb) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 100.000 | 1:LOAD CASE 1 | Max -ve | 0.000 | 0.000 | 0.000 | 249.844 |
|  |  |  |  | Max +ve | 0.000 | 0.000 |  |  |
| 2 | 2 | 100.000 | 1:LOAD CASE 1 | Max -ve | 0.000 | 0.000 |  |  |
|  |  |  |  | Max +ve | 0.000 | 0.000 | 0.000 | -213.899 |
| 3 | 3 | 100.000 | 1:LOAD CASE 1 | Max -ve | 0.000 | 0.000 | 0.000 | 250.156 |
|  |  |  |  | Max +ve | 0.000 | 0.000 |  |  |

Beam Maximum Axial Forces

Distances to maxima are given from beam end A.

| Beam | Node A | Length (in) | L/C |  | d (in) | Max Fx (lb) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 100.000 | 1:LOAD CASE 1 | Max -ve |  |  |
|  |  |  |  | Max +ve | 0.000 | -213.899 |
| 2 | 2 | 100.000 | 1:LOAD CASE 1 | Max -ve |  |  |
|  |  |  |  | Max +ve | 0.000 | -249.844 |
| 3 | 3 | 100.000 | 1:LOAD CASE 1 | Max -ve | 0.000 | 213.899 |
|  |  |  |  | Max +ve |  |  |

Beam Maximum Forces by Section Property

|  |  | Axial | Shear |  | Torsion | Bending |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Section |  | Max Fx (lb) | Max Fy (lb) | Max Fz (lb) | Max Mx (lb-in) | Max My (lb-in) | Max Mz (lb-in) |
| Rect 5.00x5.00 | Max +ve | 213.899 | 250.156 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
|  | Max -ve | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | -14.3E 3 |

Beam Combined Axial and Bending Stresses

| Beam | L/C | d | Corner 1 (psi) | Corner 2 (psi) | Corner 3 (psi) | Corner 4 (psi) | Max Tens (psi) | Max Comp (psi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1:LOAD CASE 1 | 0.000 | -694.700 | -694.700 | 677.588 | 677.588 | -694.700 | 677.588 |
|  |  | 10.000 | -574.775 | -574.775 | 557.663 | 557.663 | -574.775 | 557.663 |
|  |  | 20.000 | -454.849 | -454.849 | 437.737 | 437.737 | -454.849 | 437.737 |
|  |  | 30.000 | -334.924 | -334.924 | 317.812 | 317.812 | -334.924 | 317.812 |
|  |  | 40.000 | -214.999 | -214.999 | 197.887 | 197.887 | -214.999 | 197.887 |
|  |  | 50.000 | -95.073 | -95.073 | 77.961 | 77.961 | -95.073 | 77.961 |
|  |  | 60.000 | 24.852 | 24.852 | -41.964 | -41.964 | -41.964 | 24.852 |
|  |  | 70.000 | 144.777 | 144.777 | -161.889 | -161.889 | -161.889 | 144.777 |
|  |  | 80.000 | 264.703 | 264.703 | -281.815 | -281.815 | -281.815 | 264.703 |
|  |  | 90.000 | 384.628 | 384.628 | -401.740 | -401.740 | -401.740 | 384.628 |
|  |  | 100.000 | 504.553 | 504.553 | -521.665 | -521.665 | -521.665 | 504.553 |
| 2 | 1:LOAD CASE 1 | 0.000 | 503.116 | 503.116 | -523.103 | -523.103 | -523.103 | 503.116 |
|  |  | 10.000 | 400.444 | 400.444 | -420.431 | -420.431 | -420.431 | 400.444 |
|  |  | 20.000 | 297.772 | 297.772 | -317.760 | -317.760 | -317.760 | 297.772 |
|  |  | 30.000 | 195.101 | 195.101 | -215.088 | -215.088 | -215.088 | 195.101 |
|  |  | 40.000 | 92.429 | 92.429 | -112.417 | -112.417 | -112.417 | 92.429 |
|  |  | 50.000 | -10.243 | -10.243 | -9.745 | -9.745 | -10.243 | 0.000 |
|  |  | 60.000 | -112.914 | -112.914 | 92.927 | 92.927 | -112.914 | 92.927 |
|  |  | 70.000 | -215.586 | -215.586 | 195.598 | 195.598 | -215.586 | 195.598 |
|  |  | 80.000 | -318.258 | -318.258 | 298.270 | 298.270 | -318.258 | 298.270 |
|  |  | 90.000 | -420.929 | -420.929 | 400.942 | 400.942 | -420.929 | 400.942 |
|  |  | 100.000 | -523.601 | -523.601 | 503.613 | 503.613 | -523.601 | 503.613 |
| 3 | 1:LOAD CASE 1 | 0.000 | -505.051 | -505.051 | 522.163 | 522.163 | -505.051 | 522.163 |
|  |  | 10.000 | -384.976 | -384.976 | 402.088 | 402.088 | -384.976 | 402.088 |
|  |  | 20.000 | -264.902 | -264.902 | 282.014 | 282.014 | -264.902 | 282.014 |
|  |  | 30.000 | -144.827 | -144.827 | 161.939 | 161.939 | -144.827 | 161.939 |
|  |  | 40.000 | -24.752 | -24.752 | 41.864 | 41.864 | -24.752 | 41.864 |
|  |  | 50.000 | 95.322 | 95.322 | -78.210 | -78.210 | -78.210 | 95.322 |
|  |  | 60.000 | 215.397 | 215.397 | -198.285 | -198.285 | -198.285 | 215.397 |
|  |  | 70.000 | 335.472 | 335.472 | -318.360 | -318.360 | -318.360 | 335.472 |
|  |  | 80.000 | 455.546 | 455.546 | -438.434 | -438.434 | -438.434 | 455.546 |
|  |  | 90.000 | 575.621 | 575.621 | -558.509 | -558.509 | -558.509 | 575.621 |
|  |  | 100.000 | 695.696 | 695.696 | -678.584 | -678.584 | -678.584 | 695.696 |

Beam Combined Axial and Bending Stresses Summary

|  |  |  | Max Comp |  |  | Max Tens |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beam | L/C | Length (in) | Stress (psi) | d (in) | Corner | Stress (psi) | d (in) | Corner |
| 1 | 1:LOAD CASE 1 | 100.000 | 677.588 | 0.000 | 3 | -694.700 | 0.000 | 1 |
| 2 | 1:LOAD CASE 1 | 100.000 | 503.613 | 100.000 | 3 | -523.601 | 100.000 | 1 |
| 3 | 1:LOAD CASE 1 | 100.000 | 695.696 | 100.000 | 1 | -678.584 | 100.000 | 3 |

Beam Profile Stress

There is no data of this type.

Reactions

|  |  | Horizontal | Vertical | Horizontal | Moment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Node | L/C | FX (lb) | FY (lb) | FZ (lb) | MX (lb-in) | MY (lb-in) | MZ (lb-in) |
| 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| 4 | 1:LOAD CASE 1 | -250.156 | 213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |

Reaction Summary

|  |  |  | Horizontal | Vertical | Horizontal | Moment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Node | L/C | FX (lb) | FY (lb) | FZ (lb) | MX (lb-in) | MY (lb-in) | MZ (lb-in) |
| Max FX | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min FX | 4 | 1:LOAD CASE 1 | -250.156 | 213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max FY | 4 | 1:LOAD CASE 1 | -250.156 | 213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min FY | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max FZ | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min FZ | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max MX | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min MX | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max MY | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min MY | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Max MZ | 4 | 1:LOAD CASE 1 | -250.156 | 213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| Min MZ | 1 | 1:LOAD CASE 1 | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |

Reaction Envelope

|  |  | Horizontal | Vertical | Horizontal | Moment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Node | Env | FX (lb) | FY (lb) | FZ (lb) | MX (lb-in) | MY (lb-in) | MZ (lb-in) |
| 1 | +ve | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| 1 | +ve | - | - | - | - | - | Load: 1 |
| 1 | -ve | -249.844 | -213.899 | 0.000 | 0.000 | 0.000 | 0.000 |
| 1 | -ve | Load: 1 | Load: 1 | - | - | - | - |
| 4 | +ve | 0.000 | 213.899 | 0.000 | 0.000 | 0.000 | 14.3E 3 |
| 4 | +ve | - | Load: 1 | - | - | - | Load: 1 |
| 4 | -ve | -250.156 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| 4 | -ve | Load: 1 | - | - | - | - | - |

Utilization Ratio

| Beam | Analysis Property | Design Property | Actual Ratio | Allowable Ratio | Ratio (Act./Allow.) | Clause | L/C | Ax (in2) | Iz (in4) | Iy (in4) | Ix (in4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Rect 5.00x5.00 | 5X 5 | 0.000 | 1.000 | 0.000 |  |  | 25.000 | 52.083 | 52.083 | 88.021 |
| 2 | Rect 5.00x5.00 | 5X 5 | 0.000 | 1.000 | 0.000 |  |  | 25.000 | 52.083 | 52.083 | 88.021 |
| 3 | Rect 5.00x5.00 | 5X 5 | 0.000 | 1.000 | 0.000 |  |  | 25.000 | 52.083 | 52.083 | 88.021 |

Failed Members

There is no data of this type.

Base Pressure

| Node | L/C | FX (psi) | FY (psi) | FZ (psi) |
| --- | --- | --- | --- | --- |
| 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 |
| 4 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 |

Base Pressure Summary

|  | Node | L/C | FX (psi) | FY (psi) | FZ (psi) |
| --- | --- | --- | --- | --- | --- |
| Max FX | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 |
| Min FX | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 |
| Max FY | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 |
| Min FY | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 |
| Max FZ | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 |
| Min FZ | 1 | 1:LOAD CASE 1 | 0.000 | 0.000 | 0.000 |

Statics Check Results

| L/C |  | FX (lb) | FY (lb) | FZ (lb) | MX (lb-in) | MY (lb-in) | MZ (lb-in) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1:LOAD CASE 1 | Loads | 500.000 | 0.000 | 0.000 | 0.000 | 0.000 | -50E 3 |
| 1:LOAD CASE 1 | Reactions | -500.000 | 0.000 | 0.000 | 0.000 | 0.000 | 50E 3 |
|  | Difference | -0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |